| SOCIETY OF ALLIED WEIGHT ENGINEERS, INC. | RECOMMENDED PRACTICE Document No. SAWE WD RP O-7:202x-04 |
| --- | --- |
| Aerospace • Marine • Offshore • Land Vehicle • Allied Industries Executive Director 375 Redondo Ave, #624 Long Beach, CA 90814 | Date Issued 10 June 2020 |

Blue text to be revised by committee

yellow highlighted text to be updated with each revision

Brown field (in-service) weight control

| Revision Letter | - |
| --- | --- |

Prepared by Offshore Industry Committee Standards & Practices Committee Society of Allied Weight Engineers, Inc. (SAWE, www.sawe.org)

© 202x, Society of Allied Weight Engineers, Inc

SAWE RECOMMENDED PRACTICE

Change Record

All substantive changes to the current revision of this document are identified by a solid black vertical line on the left border of the text. A summary of those changes is detailed below.

The revisions shown here are only for approved revisions of the [YYYY] copyright year. The change record history for previous revision years should be referenced in the documents from those years, if applicable.

| Rev | Date | Description | Entered by |
| --- | --- | --- | --- |
| - | 04 Mar 2020 | Initial Issue of template |  |
| 2019 | 17 July 2019 | Revised address for SAWE in template |  |
| WD-01 | 7 Feb 2020 | Initial working Draft | A Schuster |
| WD-02 | 27 Feb 2020 | Revised to reflect ABS GUIDANCE NOTES ON IN-SERVICE HULL STABILITY VERIFICATION | A Schuster |
| WD-03 | 4 March 2020 | Incorporated Comments and recommended text from Bruce Zurbechan and Manuela Bucci | A Schsuter |
| WD-04 | 10 June 2020 | Section 9.1 added from Manuela Bucci and Colin Macfarlane | R.Hundl |

Foreword

[Enter your document foreword here.]  Offshore Industry Committee to make a choice

Option #1

This document shall be maintained continuously for consideration and action by the consensus body for recommended changes to any part of the document on an annual schedule from the date of its approval.

Option #2

This document shall be maintained periodically by the review of the entire document; action to revise or reaffirm it shall be on a schedule not to exceed five years from the date of its approval.

Option #3

This document shall be maintained as a stabilized document; this status shall be subject to review on a ten-year cycle from the approval or last revision of the document.

Questions regarding this document may be made by e-mail to:  STANDARDS@sawe.org.

Disclaimer

SAWE Recommended Practices and Standards are copyrighted by the Society of Allied Weight Engineers, (SAWE) Inc., 375 Redondo Ave, #624, Long Beach, CA 90814 USA. All rights reserved. All SAWE technical reports, including standards applied and practices recommended, are advisory only. The use of this document by anyone engaged in industry or trade is entirely voluntary. There is no agreement to adhere to any SAWE standard or recommended practice, and no commitment to conform to or be guided by any technical report. Prospective users of the report are responsible for protecting themselves against liability for infringement of patents, import/export controls, etc. If this recommended practice is incorporated into a contract, it shall be binding to the extent specified in the contract.

Table of Contents

1	Scope	1

2	Purpose	1

3	Associated Documents	1

4	Definitions, Abbreviations, Acronyms	1

4.1	Definition of Terms	1

4.2	Abbreviations and Acronyms	2

5	In-Service Weight Control Methods	2

5.1	Reactive	2

5.2	Pro Active	2

5.3	Guidance	3

5.4	Concept	3

6	EDC Phase Activities	4

6.1	FEED	4

6.2	Detailed Design	4

6.3	Construction	5

7	Verification of Alteration	5

8	Configuration control,	6

9	Methods,	6

10	Reporting	7

11	References	7

Appendix A	ABS & ISO References	A-1

A.1	ABS Guidance	A-1

A.2	ISO 19901 Guidance	A-2

A.3	ISO/DIS 19901-5 2019(x), Revision	A-18

List of Figures

No table of contents entries found.

List of Tables

Table 4-1.  Abbreviations and Acronyms	2

# Scope

This RP is intended to compliment Class, ISO Standard and Regulatory requirements, with some “how to get it done” guidance.

# Purpose

This document contains guidance for project engineers, managers, weight control engineers,  vessel’s marine technical authorities and the crew on the action needed to Control the Weight of FPU, floater or Unit while in-service so that the unit’s weight is continuously tracked  to ensure that the hydrostatic stiffness of the unit is acceptable at all times. This is recommended practices to integrate [????]

# Associated Documents

This recommended practice shall be used in conjunction with the following publications. When the following specifications are superseded by an approved revision, the revision shall apply.

| SAWE RP 0-1,2015 | Offshore Terminology |
| --- | --- |

# Definitions, Abbreviations, Acronyms

## Definition of Terms

Common Offshore Industry Weight Control terminology can be found in SAWE RP O-1 Terminology.

For the purposes of this working draft these terms are differentiated.

Weight Control

The process of setting a budget, tracking the design verse the budget and then verifying the actual weight with one or more tests.  Corrective action is initiated by the Weight Control Lead when design or actuals vary compare to the budget.

For a unit in service (not a new build) it is the process of setting the datum (a starting point that is known by measurement or as a target), tracking changes, assessing whether they are significant enough to require action or change of the datum and confirming them with one or more measurement tests.

Weight Management

A passive version of Weight Control, where the Project Engineer or Manager, or the Marine Technical Authority of the unit initiates corrective action, if any action is taken all.

Weight Control Engineer

The person responsible for the record and tracking of onboard weights.  On the vessel in operation the role may be given to a crew member so that weight changes are immediately observed and tracked with awareness of the purpose of the exercise.

Weight Change

Every item that is not part of the weight budget (defined at the design and construction stages) or that is not part of the datum weight.  To be identified by a descriptive name, weight and position onboard the unit in three coordinates in the vessel’s reference system.

## Abbreviations and Acronyms

Abbreviations and acronyms used in this recommended practice are listed in Table 4-1-.

Table 4-1.  Abbreviations and Acronyms

| Symbol | Description |
| --- | --- |
| ABS | American Bureau of Shipping ABS Plaza, 16855 Northchase Drive, Houston, TX 77060 USA Eagle.org |  |
| DWT | Deadweight |  |
| FEED | Front End Engineering & Design |
| WCPLCG | Weight Control PlanLongitudinal Centre of Gravity |
| TCG | Transverse Centre of Gravity |
| TPI | Tons Per Inch Immersion |
| VCG | Vertical Centre of Gravity |
| WCP | Weight Control Plan |

# In-Service Weight Control Methods

Weight control targets, independently, the deadweight and lightweight.

Guidelines to track changes in the lightweight are published by several Authorities and Institutions.

No guideline to monitor deadweight is, however, provided to help the crew to input the correct entries in the loading program when assessing the vessel’s stability.

There are 2 principal methods for In-Service Weight Control.

## Reactive

This method is initiated with an external mandate such as:

- Observation or report of a significant difference between the current weight and the last lightship survey weight (whether part of inclining test or not)

- Just before a class mandated lightship survey

- Pre-Alteration FEED study to confirm that the new project can be accommodated on the unit without violating any stability limits.

## Pro Active

This method starts in early stage design with the creation of an In-Service Weight Control Plan. This WCP overlaps the engineering and construction version and ends at decommissioning. The engineering and construction WCP ends with a lightweight survey that sets the start (datum) of the In-Service WCP that ends at decommissioning.

The In-Service WCP is recalibrated with a new datum after every lightship survey or when continuous measurements are collated and, if possible, after a maximum allowable threshold of weight change is exceeded.  The maximum allowable threshold of weight change is defined later in section 9.

The In-Service WCP is used by the Operator, and contractors to coordinate efforts and to assure action is taken as needed to control the weight and center of gravity of the Offshore vessel.

## Guidance

The ABS Guidance Notes on In-Service Hull Stability Verification call for a Weight Management to record any changes to lightweight due to changes which  should be recorded in a lightship data alteration log. (See appendix A for a copy of the relevant sections)

The ISO 19901-2016 Standard has an Informative Appendix G, that provides extensive guidelines on how to prepare In-Service Weight control during operations.

The ISO/DIS 19901-5 2019(x), Revision, section 7 provides a short version 2016 Appendix G, (see appendix A for a copy of the relevant informative appendix).

From other industries we know that the best way to control weight is to estimate, calculate and weigh as many of the components as possible, and to audit normal operations to prevent undefined (i.e. phantom) weight changes.  For example, some ships are prevented from ordering paint or buying welding supplies, much less new equipment, in order to control weight.

## Concept

The displacement and weight of an Offshore vessel will change In-Service.  It is important to identify type, magnitude and impact of the changes, before starting either of methods described above. The weight control engineer and operator should plan for these changes to occur and adjust the onboard lightship weight and center when the changes occur.  The list below is based on experience from several Offshore vessels and is indicative rather than comprehensive.

- Future Safety Reserve – for future enhancements required by class, regulatory bodies or charter parties.

- Future Facility Expansion Reserve – scenarios for adding additional trains, flowlines, secondary recovery systems

- Renovation Reserve - for items left on the FPU when other changes are made because it is too expensive to remove (foundations, lifting pads, etc.). To this group also belong those items of deadweight that, over time, are confused with lightweight (large machinery, third party equipment that has always been onboard, etc).

- Unit’s deterioration – Over time the unit will deteriorate.  Remedial action such as painting will add weight.  Corrosion or cathodic action of anodes will reduce weight, the position of weight and displacement. These should be assessed to determine the significance, based on the detailed design engineering models.

- Variable weights – the deadweight is recorded onboard in the loading condition selected by the crew for stability assessment. The majority of the entries are liquids in tanks and cargo or payload items and tend to be as accurate as possible. However, there are certain entries that are never updated and may add significant uncertainty to the vessel weight. These are usually stores, spares, provisions, workshops and warehouse stores that are considered as a constant in the stability assessment since their first entry input by the designer (these are the common entries: “provisions and stores”). Depending on the type of vessel the magnitude of the weight and relevant change can be significant.

# EDC Phase Activities

## FEED

An In-Service Weight Control concept is usually identified during FEED.  In most cases the Contractor’s responsibility ends with installation or startup, which means the Operator is responsible for identifying how weight will be controlled.  The concept will be different for a unit with 200 staff and a periodically “unmanned” platform.

The important point is that some sort of In-Service Weight Control Plan should be identified.  It may be a part of a larger Project or Operating Plan for the Unit.

## Detailed Design

During detailed Design several things should be done while information and discipline engineers are available to help

- Assure that weight report or database details can be decipher or interpreted to identify parts in a system.  For example, piping system components are best reported in a sequence from the storage tank to the point of application.  Structure is best reported consistently from north to south or forward to aft or from the bottom to top of a column.  This will make identifying bulk changes easier and accessible by each specific department.

- Equipment is best identified by location, so that the motor, controller, remote read out are identified separately in the database by with location and system code, to allow changes to be identified.  This is especially important in terms of vertical position for floating units.

- Identify potential changes to the unit over the life cycle. Things will change, due to wear and tear or need to be replaced as they becomes obsolete. Some examples include:

  - anode wastage

  - paint accumulation

  - more modern controllers or equipment

  - new materials to reduce maintenance, but not included in the construction

  - Secondary recovery systems

  - future or anticipated safety upgrades to systems

  - facility expansion additional trains or drilling systems

  - This stage affects the lightweight only.

## Construction

During Construction weights change to suit supplies, accessories, shipyard standards, manufacture etc.   This is similar to Detailed Design

- Assure that weight reporting and database details can be deciphered or interpreted to identify parts in a system.

- Equipment is best identified by location, so that the motor, controller, remote read out are identified separate in the database by with location and system code, to allow changes to be identified.

The total weight change is to be communicated to the designer so that its effect on the design can be analyzed before vessel delivery.

This stage affects the lightweight only.

## Operating Life

An In-Service Weight Control Plan should be completed, reviewed, accepted by the Operator and any class/regulatory authority before the unit is installed.  It is best to implement the WCP immediately after the Inclining Test or weighing of the Topsides to capture any changes.  Again, the WCP could be a part of a larger operating plan for the unit

Like any good WCP it is best to include:

- Scope/Purpose

- Description of the unit

- Definitions of terms/abbreviations

- Organization and Administration of the Plan

- How the onboard personnel are to receive Weight Updates in a Timely manner

- Roles and Responsibilities of key personnel

- Weight Control or Stability limits and the impact of significant changes

- How the weight change of alterations or changes are controlled (i.e. estimated, calculated, weighed)

- How the total weight change of all alterations are tracked and verified with In-Service Stability or Light Ship weight Surveys.

At this stage the deadweight measuring and tracking start to play a crucial role.

While lightweight changes can be tracked at discrete moments during the unit’s life, the actual deadweight is generally recorded by the crew on a daily basis to compile the daily loading condition for stability assessment. However, some of the deadweight entries are commonly replicated without any check; these are:

- Stores

- Warehouses

- Workshops

- Provisions

- Spares

- Miscellaneous (all those entries that are not liquids in tanks, cargo, payload)

A good In-Service weight control includes tracking of such weights.

# Verification of Alteration

Ideally, the weight and center of gravity location of all alterations and components weighed by suppliers for the alteration are identified in an “AS-BUILT” CAD model and installed exactly as modeled.

The ideal is very seldom the case, so a bit of practical Weight Control.

- Weighed weights of components from similar project can be used

- The weight of structure can be tracked by having the shore staff weigh what is sent out and returned.  This may require separating the scrap material shore side by bulk type.

- The location of controllers, starters, spares, maintenance supplies are tracked based on interviews with operating staff.

Verification of actual weight change is possible onboard a floating unit by comparison between the calculated unit weight (with the onboard loading program) and the measured draughts (calibrated onboard draught gauges or other reliable devices).  Discrepancy is commonly identified as ‘phantom weight’. A recurrent phantom weight is an indicator of high uncertainty in the recorded weight change.

The comparison between calculated and measured draughts is to be recorded at each input of weight change.

# Configuration control,

The configuration of the unit can be confirmed by inspections by the Weight Control Engineer or with a review of unit paperwork.  The Weight Control Engineer is typical a part of any Engineering Change Proposal review and approval cycle for the operator.

The Weight Control Engineer usually has a list of changes made to the unit since the last inclining test or weighing.

The configuration control inspection shall be repeated once a month at a minimum or as soon as a change is applied as planned and involves update of both lightweight and deadweight records.

# Methods,

There are several methods to monitor the weight changes that can be used:

- Annual Surveys by Discipline Engineers usually include photos or equipment status reports.  These can be perused to confirm no changes have been made.

- Vet by the Weight Control Engineer to the unit on a 12 to 36 month basis is another method to confirm configuration changes.

- Ranking of weight and displacement changes so that quick assessments can be made of what changes are important. Typically every change is ranked on a scale of  -5 to +5  where -5 and +5 are significant weight or moment impacts and +/-1 is insignificant. Once a significant number of changes have been identified (i.e. items greater than +/ - 1) weight estimates, calculations and weighings are initiated.

- There is a lot written on what is insignificant but a first cut estimate is 10% of the TPI in the operating condition for single items or group items (such as an assembled piping and valves cluster composed by several lighter items).  While a significant weight change would be  100% of an inch of TPI.  The effect of a weight change is typically expressed in the added weight to level the unit for a weight change plus the weight change.

- However, the impact on the stability of the unit is ruled by the vertical moment due to the weight change.

  - In recording the weight change, the centre of gravity (LCG, TCG and VCG) of each entry shall be recorded and the actual position of the total weight change is to be calculated

LCG=(wi×LCGi)wi  		TCG=(wi×VCGi)wi	VCG=(wi×VCGi)wi

- The total weight change, calculated as sum of all record entries with their sign, is to be input in the onboard loading program with weight and position onboard. In this way, the effect on the overall stability is always tracked.  It is important that removed weight is also treated with a negative sign (added weights with positive sign), since the overall effect might be a negligible weight change but a significant change of centre of gravity, i.e. equilibrium or stability of the unit.

## How In-Service Weighing Should Be Done

### Lightship

A table with the following entries should be made available onboard since the entry in service:

- Date

- Identifier

- Weight

- Location (LCG, TCG, VCG of the entry in the vessel’s reference system)

- Longitudinal, transverse and vertical moment

- Whether this weight is measured, known from documentation or estimated and the source of information on the weight

- Uncertainty

For the total weight change and for the lightship, the weight, coordinates of center of gravity and moments should be updated every time new changes are input.

An entry “Lightship changes [date]” should be set in the onboard loading program and it should be updated every time the total change is such as to exceed the vessel’s TPI or 2.5 times TPCm (the sinkage value TPI/TPCm at the operation draught is suggested, but different reference draughts might be chosen for a specific Offshore Unit).

Most of the Offshore Units already keep an updated record of this kind and it is a requirement of the IMO MODU Code at 3.1.4 “A record of all changes to machinery, structure, outfitting and equipment that affect the light ship data should be maintained in a light ship data alterations log and be taken into account in daily operations”.  Other Institutions refer to this type of record (e.g. USCG, ABS).

An example of a typical form to record lightship weight changes is in Figure 1.

Guidelines for the calculation of uncertainties are in the Section 16 of the SAWE Handbook.

Figure 1 - Example of record of lightship changes

### Deadweight

The deadweight is generally input in the loading program according to information obtained by the responsible authority during daily inspections.  These inspections generally focus on the Variable Deck Load (VDL – temporary equipment or machinery used for onboard operations, container modules, third party equipment, non-structural tanks).

Tank content is uploaded, either, directly from the onboard gauging system, or from manual sounding – this is generally completed on a daily basis.

All other deadweight such as, for example, the content of stores, warehouses, workshops, etc, is generally input in the loading condition with a constant value as it was recorded at the last special survey.

The above method leaves two main gaps in accurately tracking the onboard weight configuration:

- Liquid residuals at tank bottom are neglected. This weight is onboard, but when tanks are nominally empty, it is still there but is neglected

- Weight changes in stores, warehouses, workshops, etc, are not updated

This lack of knowledge adds uncertainty to the displacement calculated by the onboard loading program.

The following should be recorded:

- The typical liquid remaining should be recorded at the first tank inspection, that is after the tanks have been used and normally emptied by the onboard stripping system. It is recommended to repeat the measurements during the first years of service and update the quantity of liquid remaining with the average of the updated record. Nominally empty tanks should be set with the weight of the liquid remaining.

- At the same time as the tank is ‘characterized’ in terms of residual fluid, there should also be recorded the position of inlets, discharges, air pipes and sounding pipes or gauges. This information to be used to account for heel and trim when calculating the tanks content from measurement of fluid level into a tank.

- Spaces containing a significant magnitude of deadweight should be regularly reviewed, in order to include the most realistic weight in the loading program. In order to reduce the task to the significant areas only, emphasis is to be given to spaces with a content weight that is significant (for example the quantity TPCT, where T is the uncertainty in the draught measurement may be used as threshold of relevance, any weight larger than the threshold is relevant).  These spaces should be reviewed in the daily inspection and, in any case, at intervals not longer than one week.

This task can be simplified by making available on-site a list of items and weights on shelves/in rooms so that every addition/removal can be marked by manual notes. For the control operator who reviews the daily loading condition, it should be easier to collect the updated information.

- The uncertainty should be calculated and included in the loading condition – For this case acceptability may be defined for total uncertainty in weight < TPCT, where T is the uncertainty in the draught measurement.  Guidelines for uncertainty calculation are in Section 16 of the SAWE Handbook.

### Check Record

A tool for recording calculated draughts (from the loading program) and measured draughts should be available onboard.  This is commonly included within the functionality of the onboard loading program.  If not, a separate system (spreadsheet or similar) should be made available that is easy to use.  An example of such a record is shown in Figure 2.

For every loading condition uploaded in the onboard loading program, the calculated mean draught and the measured draught (from the onboard gauging system) should be recorded and the difference calculated.

Discrepancies are indicators of (any of the following or their combination):

- Unrecorded change in lightship

- Error in the hydrostatic model used to calculate draughts

- Non-calibrated draught gauges

- Inaccurate deadweight

In particular, the first two tend to show recurrent discrepancy with same sign (the vessel is calculated always heavier or always lighter), while the second two tend to be irregular in magnitude and sign.

A plot of the time series of the differences can provide a quick diagnosis of the source of ghost weights.

Figure 2 - Example of record of draught check

Onboard gauges should always be operational and calibrated as per good practice

Small differences, however, might be due to uncertainty of the measured draughts or uncertainty in the calculated draughts.  A difference of mean draught that corresponds to a weight difference larger than 3TPC (metric) or TPI (imperial) should be classified as ghost weight.

# Reporting

An Annual Weight Report for the unit is recommended for to the Operator’s On-Site Inspection team to confirm that the weight changes recorded in the Office and Onboard match.  Discrepancies are usually worked out in a timely manner.  Ranking the magnitude of the discrepancies is a useful tool to prioritize the work.

Deadweight should be always updated.  It is responsibility of the crew to review and revise also such entries that are less likely to change but which must be as accurate as possible.

Most Marine Operating Manuals (MOM) require regular weight reporting. This is usually done with the Load Management Software (LMS), but in the MOM there are manual calculation forms if the power fails. The MOM spells out what to do if there is a discrepancy.

# References

[1]	ABS, GUIDANCE NOTES ON IN-SERVICE HULL STABILITY VERIFICATION

[2] 	ISO 19901-5, 2016, Petroleum and natural gas industries — Specific requirements for offshore structures — Part 5: Weight control during engineering and construction

###### ABS & ISO References

The following have been provided for the Working Draft only, and will be removed in final version of the working draft.

ABS Guidance

ISO 19901 Guidance

ISO/DIS 19901-5 2019(x), Revision

MTN 04-95, CH-2 16710/Lightship Change January 11, 2016

ABS Information Sheet “Weight Control for Floating Offshore Installations”, 2005

The uncertainty of the draught measurement is defined by the resolution of the measuring device.

Onboard gauges should always be operational and calibrated as per good practice

Small differences, however, might be due to uncertainty of the measured draughts or uncertainty in the calculated draughts.  A difference of mean draught that corresponds to a weight difference larger than 3TPC (metric) or TPI (imperial) should be classified as ghost weight.